IEC Standard Caledonian Offshore & Marine Cables

MariSig Flame Retardant Instrumentation & Control Cables

www.caledonian-cables.co.uk

MRE-2XH 150/250V XLPE Insulated, LSOH (SHF1) Sheathed Flame Retardant Instrumentation & Control Cables (Multicore)

Application

These cables are used on board of ships at all locations for fixed installations not subject to mechanical risk complying with IEC standards 60092-352. These cables are flame retardant, low smoke & halogen free, suitable for installations on passenger ships, as on other commercial vessels.

Standards

	IEC 60092-350/351/376/359

	IEC 60332-1

	IEC 60332-3-22

	IEC 60754-1/2

	IEC 61034

Stranded Copper Conductor XLPE Insulation

LSOH (SHF1) Sheath

Construction

- Conductors: Class 2 stranded copper conductor.

- Insulation: XLPE.

- Outer Sheath: LSOH (SHF1). SHF2 can be offered upon request.

Core Identification

White with printed number.

Mechanical and Thermal Properties

Bending Radius for Fixed Installations: 4×OD Temperature Range: -30℃ ~ +80℃

44	ADDISON	www.addison-cables.com

IEC Standard Caledonian Offshore & Marine Cables

MariSig Flame Retardant Instrumentation & Control Cables

www.caledonian-cables.co.uk

Dimensions and Weight

| Part No. | Construction No. of cores×Cross section(mm²) | Nominal Insulation Thickness mm | Nominal Sheath Thickness mm | Nominal Overall Diameter mm | Nominal Weight kg/km |
| --- | --- | --- | --- | --- | --- |
| MRE-2XH-2C0.75 | 2×0.75 | 0.5 | 1.1 | 6.2 | 50 |
| MRE-2XH-3C0.75 | 3×0.75 | 0.5 | 1.1 | 6.5 | 60 |
| MRE-2XH-5C0.75 | 5×0.75 | 0.5 | 1.2 | 7.7 | 80 |
| MRE-2XH-7C0.75 | 7×0.75 | 0.5 | 1.2 | 8.3 | 100 |
| MRE-2XH-12C0.75 | 12×0.75 | 0.5 | 1.3 | 10.9 | 170 |
| MRE-2XH-19C0.75 | 19×0.75 | 0.5 | 1.6 | 13.0 | 250 |
| MRE-2XH-27C0.75 | 27×0.75 | 0.5 | 1.7 | 15.6 | 350 |
| MRE-2XH-37C0.75 | 37×0.75 | 0.5 | 1.9 | 17.6 | 470 |
| MRE-2XH-2C1.0 | 2×1.0 | 0.5 | 1.1 | 6.6 | 60 |
| MRE-2XH-3C1.0 | 3×1.0 | 0.5 | 1.2 | 7.0 | 70 |
| MRE-2XH-5C1.0 | 5×1.0 | 0.5 | 1.2 | 8.2 | 100 |
| MRE-2XH-7C1.0 | 7×1.0 | 0.5 | 1.3 | 9.1 | 130 |
| MRE-2XH-12C1.0 | 12×1.0 | 0.5 | 1.4 | 11.7 | 210 |
| MRE-2XH-19C1.0 | 19×1.0 | 0.5 | 1.7 | 14.0 | 310 |
| MRE-2XH-27C1.0 | 27×1.0 | 0.5 | 1.9 | 16.8 | 430 |
| MRE-2XH-37C1.0 | 37×1.0 | 0.5 | 2.0 | 19.0 | 580 |

www.addison-cables.com

ADDISON	45