IEC Standard Caledonian Offshore & Marine Cables

MariTox Marine Flame Retardant Medium Voltage Cables

www.caledonian-cables.co.uk

MTX 3.6/6kV, 6/10kV, 8.7/15kV XLPE Insulated, LSOH (SHF1)

Sheathed, Armoured Flame Retardant MV Power Cables (SHF1 Inner Sheath)

Application

These armoured MV cables are used on board of ships in all locations for fixed installations complying with IEC standards 60092-352. These cables are flame retardant, low smoke & halogen free.

Standards

	IEC 60092-350/351/354/359

	IEC 60332-1

	IEC 60332-3-22

	IEC 60754-1/2

	IEC 61034

Copper/Galvanized Steel Wire Braid Armour LSOH(SHF1) Inner Sheath

Stranded Copper Conductor Conductor Screen

XLPE Insulation Insulation Screen Metallic Screen

LSOH(SHF1) Outer Sheath

Construction

- Conductors: Class 2 stranded copper conductor.

- Conductor Screen: Semi-conducting layer (tape/compound).

- Insulation: XLPE.

- Insulation Screen: Semi-conducting layer (tape/compound).

- Metallic Screen: Copper tape

- Inner Sheath: LSOH (SHF1).

- Armour: Copper wire braid or galvanized steel wire braid.

- Outer Sheath: LSOH (SHF1). SHF2 can be offered upon request.

Core Identification

Coloured tape shall be inserted under metallic screen. 3core: Red, Yellow, Blue.

8	ADDISON	www.addison-cables.com

IEC Standard Caledonian Offshore & Marine Cables

MariTox Marine Flame Retardant Medium Voltage Cables

www.caledonian-cables.co.uk

Mechanical and Thermal Properties

Bending Radius for Fixed Installations: 12×OD (single core); 9×OD (three core) Temperature Range: -30℃ ~ +80℃

Dimensions and Weight

3.6/6kV

| Part No. | Construction No. of cores×Cross section(mm²) | Nominal Insulation Thickness mm | Nominal Sheath Thickness mm |  | Nominal Overall Diameter mm | Nominal Weight kg/km |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Inner | Outer |  |  |
| MTX-3.6/6KV-RZ1MZ1-1C10 | 1×10 | 2.5 | 1.3 | 1.0 | 19.2 | 560 |
| MTX-3.6/6KV-RZ1MZ1-1C16 | 1×16 | 2.5 | 1.3 | 1.0 | 20.1 | 650 |
| MTX-3.6/6KV-RZ1MZ1-1C25 | 1×25 | 2.5 | 1.4 | 1.1 | 21.8 | 800 |
| MTX-3.6/6KV-RZ1MZ1-1C35 | 1×35 | 2.5 | 1.4 | 1.1 | 23.0 | 940 |
| MTX-3.6/6KV-RZ1MZ1-1C50 | 1×50 | 2.5 | 1.5 | 1.2 | 24.7 | 1120 |
| MTX-3.6/6KV-RZ1MZ1-1C70 | 1×70 | 2.5 | 1.6 | 1.2 | 26.7 | 1390 |
| MTX-3.6/6KV-RZ1MZ1-1C95 | 1×95 | 2.5 | 1.6 | 1.3 | 28.8 | 1720 |
| MTX-3.6/6KV-RZ1MZ1-1C120 | 1×120 | 2.5 | 1.7 | 1.3 | 30.6 | 2020 |
| MTX-3.6/6KV-RZ1MZ1-1C150 | 1×150 | 2.5 | 1.8 | 1.3 | 32.4 | 2350 |
| MTX-3.6/6KV-RZ1MZ1-1C185 | 1×185 | 2.5 | 1.8 | 1.4 | 34.4 | 2780 |
| MTX-3.6/6KV-RZ1MZ1-1C240 | 1×240 | 2.6 | 2.0 | 1.5 | 38.4 | 3580 |
| MTX-3.6/6KV-RZ1MZ1-1C300 | 1×300 | 2.8 | 2.1 | 1.6 | 41.6 | 4310 |
| MTX-3.6/6KV-RZ1MZ1-1C400 | 1×400 | 3.0 | 2.2 | 1.7 | 45.8 | 5440 |
| MTX-3.6/6KV-RZ1MZ1-1C500 | 1×500 | 3.2 | 2.4 | 1.7 | 49.3 | 6450 |
| MTX-3.6/6KV-RZ1MZ1-1C630 | 1×630 | 3.2 | 2.5 | 1.9 | 54.1 | 8110 |
| MTX-3.6/6KV-RZ1MZ1-3C10 | 3×10 | 2.5 | 1.9 | 1.4 | 36.3 | 1710 |
| MTX-3.6/6KV-RZ1MZ1-3C16 | 3×16 | 2.5 | 2.0 | 1.5 | 38.8 | 2030 |
| MTX-3.6/6KV-RZ1MZ1-3C25 | 3×25 | 2.5 | 2.1 | 1.6 | 42.0 | 2500 |
| MTX-3.6/6KV-RZ1MZ1-3C35 | 3×35 | 2.5 | 2.2 | 1.6 | 44.8 | 2950 |
| MTX-3.6/6KV-RZ1MZ1-3C50 | 3×50 | 2.5 | 2.3 | 1.7 | 48.0 | 3510 |
| MTX-3.6/6KV-RZ1MZ1-3C70 | 3×70 | 2.5 | 2.5 | 1.8 | 52.5 | 4420 |
| MTX-3.6/6KV-RZ1MZ1-3C95 | 3×95 | 2.5 | 2.6 | 1.9 | 57.0 | 5470 |
| MTX-3.6/6KV-RZ1MZ1-3C120 | 3×120 | 2.5 | 2.8 | 2.0 | 61.0 | 6480 |
| MTX-3.6/6KV-RZ1MZ1-3C150 | 3×150 | 2.5 | 2.9 | 2.1 | 64.9 | 7540 |
| MTX-3.6/6KV-RZ1MZ1-3C185 | 3×185 | 2.5 | 3.1 | 2.2 | 69.3 | 8940 |
| MTX-3.6/6KV-RZ1MZ1-3C240 | 3×240 | 2.6 | 3.3 | 2.4 | 76.4 | 11180 |

6/10kV

| Part No. | Construction No. of cores×Cross section(mm²) | Nominal Insulation Thickness mm | Nominal Sheath Thickness mm |  | Nominal Overall Diameter mm | Nominal Weight kg/km |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Inner | Outer |  |  |
| MTX-6/10KV-RZ1MZ1-1C16 | 1×16 | 3.4 | 1.4 | 1.1 | 22.3 | 750 |
| MTX-6/10KV-RZ1MZ1-1C25 | 1×25 | 3.4 | 1.5 | 1.1 | 23.8 | 890 |
| MTX-6/10KV-RZ1MZ1-1C35 | 1×35 | 3.4 | 1.5 | 1.2 | 25.2 | 1040 |
| MTX-6/10KV-RZ1MZ1-1C50 | 1×50 | 3.4 | 1.6 | 1.2 | 26.7 | 1220 |
| MTX-6/10KV-RZ1MZ1-1C70 | 1×70 | 3.4 | 1.6 | 1.2 | 28.5 | 1480 |

www.addison-cables.com

ADDISON	9

IEC Standard Caledonian Offshore & Marine Cables

MariTox Marine Flame Retardant Medium Voltage Cables

www.caledonian-cables.co.uk

| Part No. | Construction No. of cores×Cross section(mm²) | Nominal Insulation Thickness mm | Nominal Sheath Thickness mm |  | Nominal Overall Diameter mm | Nominal Weight kg/km |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Inner | Outer |  |  |
| MTX-6/10KV-RZ1MZ1-1C95 | 1×95 | 3.4 | 1.7 | 1.3 | 30.8 | 1830 |
| MTX-6/10KV-RZ1MZ1-1C120 | 1×120 | 3.4 | 1.8 | 1.3 | 32.6 | 2140 |
| MTX-6/10KV-RZ1MZ1-1C150 | 1×150 | 3.4 | 1.8 | 1.4 | 34.4 | 2470 |
| MTX-6/10KV-RZ1MZ1-1C185 | 1×185 | 3.4 | 1.9 | 1.4 | 36.9 | 2990 |
| MTX-6/10KV-RZ1MZ1-1C240 | 1×240 | 3.4 | 2.0 | 1.5 | 40.0 | 3680 |
| MTX-6/10KV-RZ1MZ1-1C300 | 1×300 | 3.4 | 2.1 | 1.6 | 42.8 | 4390 |
| MTX-6/10KV-RZ1MZ1-1C400 | 1×400 | 3.4 | 2.3 | 1.7 | 46.8 | 5510 |
| MTX-6/10KV-RZ1MZ1-1C500 | 1×500 | 3.4 | 2.4 | 1.8 | 49.9 | 6480 |
| MTX-6/10KV-RZ1MZ1-1C630 | 1×630 | 3.4 | 2.5 | 1.9 | 54.5 | 8130 |
| MTX-6/10KV-RZ1MZ1-3C16 | 3×16 | 3.4 | 2.1 | 1.6 | 43.1 | 2350 |
| MTX-6/10KV-RZ1MZ1-3C25 | 3×25 | 3.4 | 2.2 | 1.7 | 46.3 | 2840 |
| MTX-6/10KV-RZ1MZ1-3C35 | 3×35 | 3.4 | 2.3 | 1.7 | 49.1 | 3310 |
| MTX-6/10KV-RZ1MZ1-3C50 | 3×50 | 3.4 | 2.5 | 1.8 | 52.5 | 3910 |
| MTX-6/10KV-RZ1MZ1-3C70 | 3×70 | 3.4 | 2.6 | 1.9 | 56.8 | 4820 |
| MTX-6/10KV-RZ1MZ1-3C95 | 3×95 | 3.4 | 2.8 | 2.0 | 61.4 | 5930 |
| MTX-6/10KV-RZ1MZ1-3C120 | 3×120 | 3.4 | 2.9 | 2.1 | 65.3 | 6940 |
| MTX-6/10KV-RZ1MZ1-3C150 | 3×150 | 3.4 | 3.1 | 2.2 | 69.3 | 8050 |
| MTX-6/10KV-RZ1MZ1-3C185 | 3×185 | 3.4 | 3.2 | 2.3 | 73.6 | 9460 |
| MTX-6/10KV-RZ1MZ1-3C240 | 3×240 | 3.4 | 3.4 | 2.5 | 80.2 | 11680 |

8.7/15kV

| Part No. | Construction No. of cores×Cross section(mm²) | Nominal Insulation Thickness mm | Nominal Sheath Thickness mm |  | Nominal Overall Diameter mm | Nominal Weight kg/km |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Inner | Outer |  |  |
| MTX-8.7/15KV-RZ1MZ1-1C25 | 1×25 | 4.5 | 1.6 | 1.2 | 26.4 | 1030 |
| MTX-8.7/15KV-RZ1MZ1-1C35 | 1×35 | 4.5 | 1.6 | 1.2 | 27.6 | 1180 |
| MTX-8.7/15KV-RZ1MZ1-1C50 | 1×50 | 4.5 | 1.7 | 1.3 | 29.3 | 1370 |
| MTX-8.7/15KV-RZ1MZ1-1C70 | 1×70 | 4.5 | 1.7 | 1.3 | 31.1 | 1650 |
| MTX-8.7/15KV-RZ1MZ1-1C95 | 1×95 | 4.5 | 1.8 | 1.4 | 33.4 | 2010 |
| MTX-8.7/15KV-RZ1MZ1-1C120 | 1×120 | 4.5 | 1.9 | 1.4 | 35.7 | 2410 |
| MTX-8.7/15KV-RZ1MZ1-1C150 | 1×150 | 4.5 | 1.9 | 1.5 | 37.5 | 2760 |
| MTX-8.7/15KV-RZ1MZ1-1C185 | 1×185 | 4.5 | 2.0 | 1.5 | 39.5 | 3200 |
| MTX-8.7/15KV-RZ1MZ1-1C240 | 1×240 | 4.5 | 2.1 | 1.6 | 42.6 | 3910 |
| MTX-8.7/15KV-RZ1MZ1-1C300 | 1×300 | 4.5 | 2.2 | 1.6 | 45.2 | 4610 |
| MTX-8.7/15KV-RZ1MZ1-1C400 | 1×400 | 4.5 | 2.3 | 1.7 | 49.0 | 5730 |
| MTX-8.7/15KV-RZ1MZ1-1C500 | 1×500 | 4.5 | 2.5 | 1.8 | 52.3 | 6740 |
| MTX-8.7/15KV-RZ1MZ1-1C630 | 1×630 | 4.5 | 2.6 | 1.9 | 56.9 | 8400 |
| MTX-8.7/15KV-RZ1MZ1-3C25 | 3×25 | 4.5 | 2.4 | 1.8 | 51.6 | 3300 |
| MTX-8.7/15KV-RZ1MZ1-3C35 | 3×35 | 4.5 | 2.5 | 1.9 | 54.6 | 3820 |
| MTX-8.7/15KV-RZ1MZ1-3C50 | 3×50 | 4.5 | 2.6 | 1.9 | 57.6 | 4400 |
| MTX-8.7/15KV-RZ1MZ1-3C70 | 3×70 | 4.5 | 2.8 | 2.0 | 62.1 | 5370 |
| MTX-8.7/15KV-RZ1MZ1-3C95 | 3×95 | 4.5 | 3.0 | 2.2 | 67.0 | 6550 |
| MTX-8.7/15KV-RZ1MZ1-3C120 | 3×120 | 4.5 | 3.1 | 2.3 | 70.8 | 7580 |
| MTX-8.7/15KV-RZ1MZ1-3C150 | 3×150 | 4.5 | 3.2 | 2.3 | 74.5 | 8680 |
| MTX-8.7/15KV-RZ1MZ1-3C185 | 3×185 | 4.5 | 3.4 | 2.5 | 79.2 | 10180 |
| MTX-8.7/15KV-RZ1MZ1-3C240 | 3×240 | 4.5 | 3.6 | 2.6 | 85.6 | 12420 |

10	ADDISON	www.addison-cables.com