IEC Standard Caledonian Offshore & Marine Cables

MariTox Marine Flame Retardant Power & Control Cables

www.caledonian-cables.co.uk

MTX400 0.6/1 kV HEPR Insulated, LSOH (SHF1) Sheathed Flame Retardant Power & Control Cables (Multicore)

Application

These cables are used for fixed installations on ships and offshore units in all locations. These cables are flame retardant, low smoke & halogen free.

Standards

	DIN 89160/98

	IEC 60092-353

	IEC 60332-1

	IEC 60332-3-22

	IEC 60754-1/2

	IEC 61034

Stranded Copper Conductor HEPR Insulation

LSOH (SHF1) Sheath

Construction

- Conductors: Class 2 stranded copper conductor.

- Insulation: HEPR.

- Outer Sheath: LSOH (SHF1).

Core Identification

Two cores: Black, Blue.

Three cores: Black, Blue, Brown.

Four cores: Black, Blue, Brown, Black.

Five cores: Black, Blue, Brown, Black, Black.

Multi cores: All cores natural coloured, printed with numbers, starting in center with number 1. Cables with 3 cores and more also with green/yellow core (has to be stated in order).

Mechanical and Thermal Properties

Bending Radius for Fixed Installations: 6×OD (OD>25mm); 4×OD (OD ≤25mm) Temperature Range: -20℃ ~ +75℃

www.addison-cables.com

ADDISON	23

IEC Standard Caledonian Offshore & Marine Cables

MariTox Marine Flame Retardant Power & Control Cables

www.caledonian-cables.co.uk

Dimensions and Weight

| Part No. | Construction No. of cores×Cross section(mm²) | Nominal Insulation Thickness mm | Nominal Sheath Thickness mm | Nominal Overall Diameter mm | Nominal Weight kg/km |
| --- | --- | --- | --- | --- | --- |
| MTX400-1G7Z1-R-2C1.5 | 2×1.5 | 0.7 | 1.0 | 8.4 | 130 |
| MTX400-1G7Z1-R-2C2.5 | 2×2.5 | 0.7 | 1.1 | 9.3 | 170 |
| MTX400-1G7Z1-R-2C4 | 2×4 | 0.7 | 1.1 | 10.5 | 220 |
| MTX400-1G7Z1-R-2C6 | 2×6 | 0.7 | 1.2 | 11.8 | 280 |
| MTX400-1G7Z1-R-2C10 | 2×10 | 0.7 | 1.2 | 13.8 | 410 |
| MTX400-1G7Z1-R-2C16 | 2×16 | 0.7 | 1.3 | 16.2 | 590 |
| MTX400-1G7Z1-R-2C25 | 2×25 | 0.9 | 1.5 | 20.1 | 880 |
| MTX400-1G7Z1-R-3C1.5 | 3×1.5 | 0.7 | 1.1 | 8.9 | 160 |
| MTX400-1G7Z1-R-3C2.5 | 3×2.5 | 0.7 | 1.1 | 9.9 | 210 |
| MTX400-1G7Z1-R-3C4 | 3×4 | 0.7 | 1.1 | 11.1 | 270 |
| MTX400-1G7Z1-R-3C6 | 3×6 | 0.7 | 1.2 | 12.6 | 350 |
| MTX400-1G7Z1-R-3C10 | 3×10 | 0.7 | 1.3 | 14.8 | 510 |
| MTX400-1G7Z1-R-3C16 | 3×16 | 0.7 | 1.4 | 17.3 | 750 |
| MTX400-1G7Z1-R-3C25 | 3×25 | 0.9 | 1.5 | 21.5 | 1130 |
| MTX400-1G7Z1-R-3C35 | 3×35 | 0.9 | 1.6 | 24.2 | 1500 |
| MTX400-1G7Z1-R-3C50 | 3×50 | 1.0 | 1.8 | 28.5 | 1970 |
| MTX400-1G7Z1-R-3C70 | 3×70 | 1.1 | 2.0 | 32.9 | 2770 |
| MTX400-1G7Z1-R-3C95 | 3×95 | 1.1 | 2.1 | 37.4 | 3680 |
| MTX400-1G7Z1-R-3C120 | 3×120 | 1.2 | 2.3 | 41.6 | 4580 |
| MTX400-1G7Z1-R-4C1.5 | 4×1.5 | 0.7 | 1.1 | 9.8 | 200 |
| MTX400-1G7Z1-R-4C2.5 | 4×2.5 | 0.7 | 1.1 | 10.9 | 250 |
| MTX400-1G7Z1-R-4C4 | 4×4 | 0.7 | 1.2 | 12.3 | 330 |
| MTX400-1G7Z1-R-4C6 | 4×6 | 0.7 | 1.2 | 13.9 | 440 |
| MTX400-1G7Z1-R-4C10 | 4×10 | 0.7 | 1.3 | 16.3 | 660 |
| MTX400-1G7Z1-R-4C16 | 4×16 | 0.7 | 1.4 | 19.2 | 950 |
| MTX400-1G7Z1-R-4C25 | 4×25 | 0.9 | 1.6 | 23.9 | 1460 |
| MTX400-1G7Z1-R-4C35 | 4×35 | 0.9 | 1.7 | 27.0 | 1920 |
| MTX400-1G7Z1-R-4C50 | 4×50 | 1.0 | 1.9 | 30.5 | 2520 |
| MTX400-1G7Z1-R-4C70 | 4×70 | 1.1 | 2.1 | 36.7 | 3530 |
| MTX400-1G7Z1-R-4C95 | 4×95 | 1.1 | 2.3 | 41.7 | 4730 |
| MTX400-1G7Z1-R-5C1.5 | 5×1.5 | 0.7 | 1.1 | 12.3 | 230 |
| MTX400-1G7Z1-R-5C2.5 | 5×2.5 | 0.7 | 1.2 | 13.3 | 300 |
| MTX400-1G7Z1-R-7C1.5 | 7×1.5 | 0.7 | 1.2 | 13.3 | 290 |
| MTX400-1G7Z1-R-10C1.5 | 10×1.5 | 0.7 | 1.3 | 15.8 | 360 |
| MTX400-1G7Z1-R-12C1.5 | 12×1.5 | 0.7 | 1.3 | 16.3 | 410 |
| MTX400-1G7Z1-R-14C1.5 | 14×1.5 | 0.7 | 1.4 | 17.0 | 470 |
| MTX400-1G7Z1-R-16C1.5 | 16×1.5 | 0.7 | 1.4 | 18.0 | 530 |
| MTX400-1G7Z1-R-19C1.5 | 19×1.5 | 0.7 | 1.4 | 19.0 | 610 |
| MTX400-1G7Z1-R-24C1.5 | 24×1.5 | 0.7 | 1.5 | 22.0 | 760 |

24	ADDISON	www.addison-cables.com