MRE-M2XCH PiMF/TiMF 150/250V Mica Tape + XLPE Insulated, LSOH (SHF1) Sheathed, Individual Screened & Armoured Fire Resistant Instrumentation & Control Cables (Multipair/Multitriple)

Application

These cables are used on board of ships in all locations for fixed installations complying with IEC standards 60092-352 in safety circuit, where fire resistance is required. These cables are fire resistant, flame retardant, low smoke & halogen free, suitable for installations on passenger ships, as on other commercial vessels.

Standards

	IEC 60092-350/351/376/359

	IEC 60331-21

	IEC 60332-1

	IEC 60332-3-22

	IEC 60754-1/2

	IEC 61034

LSOH (SHF1) Outer Sheath Stranded Copper Conductor Mica Tape + XLPE Insulation Al/polyester Tape + Drain Wire

Polyester Tape

Copper Wire Braid Armour

Construction

- Conductors: Class 2 stranded copper conductor.

- Insulation: Mica tape + XLPE.

- Cabling Element: Pair/Triple.

- Individual Screen: Al/polyester tape.

- Drain Wire: Tinned copper wire.

- Inner Covering: Lapped polyester tape.

- Armour: Copper wire braid.

- Outer Sheath: LSOH (SHF1). SHF2 can be offered upon request.

Core Identification

Pair: White/blue with printed pair number and core number. Triple: White/blue/red with printed triple number.

Mechanical and Thermal Properties

Bending Radius for Fixed Installations: 6×OD Temperature Range: -30℃ ~ +80℃

Dimensions and Weight

| Part No. | Construction No. of elements×No. of cores in element×Cross section(mm²) | Nominal Insulation Thickness mm | Nominal Sheath Thickness mm | Nominal Overall Diameter mm | Nominal Weight kg/km |
| --- | --- | --- | --- | --- | --- |
| MRE-M2XCH PiMF-1P0.75 | 1×2×0.75 | 0.5 | 1.3 | 9.3 | 130 |
| MRE-M2XCH PiMF-2P0.75 | 2×2×0.75 | 0.5 | 1.4 | 13.6 | 230 |
| MRE-M2XCH PiMF-3P0.75 | 3×2×0.75 | 0.5 | 1.8 | 15.1 | 320 |
| MRE-M2XCH PiMF-4P0.75 | 4×2×0.75 | 0.5 | 1.8 | 16.0 | 370 |
| MRE-M2XCH PiMF-5P0.75 | 5×2×0.75 | 0.5 | 1.9 | 17.9 | 450 |
| MRE-M2XCH PiMF-6P0.75 | 6×2×0.75 | 0.5 | 1.9 | 19.1 | 510 |
| MRE-M2XCH PiMF-7P0.75 | 7×2×0.75 | 0.5 | 1.9 | 19.1 | 540 |
| MRE-M2XCH PiMF-8P0.75 | 8×2×0.75 | 0.5 | 2.0 | 20.4 | 610 |
| MRE-M2XCH PiMF-10P0.75 | 10×2×0.75 | 0.5 | 2.1 | 22.9 | 740 |
| MRE-M2XCH PiMF-12P0.75 | 12×2×0.75 | 0.5 | 2.1 | 23.7 | 830 |
| MRE-M2XCH PiMF-14P0.75 | 14×2×0.75 | 0.5 | 2.1 | 24.6 | 910 |
| MRE-M2XCH PiMF-16P0.75 | 16×2×0.75 | 0.5 | 2.2 | 26.5 | 1040 |
| MRE-M2XCH PiMF-19P0.75 | 19×2×0.75 | 0.5 | 2.3 | 28.4 | 1200 |
| MRE-M2XCH PiMF-20P0.75 | 20×2×0.75 | 0.5 | 2.3 | 28.4 | 1230 |
| MRE-M2XCH PiMF-24P0.75 | 24×2×0.75 | 0.5 | 2.4 | 32.3 | 1490 |
| MRE-M2XCH PiMF-30P0.75 | 30×2×0.75 | 0.5 | 2.5 | 35.1 | 1790 |
| MRE-M2XCH PiMF-37P0.75 | 37×2×0.75 | 0.5 | 2.8 | 38.1 | 2200 |
| MRE-M2XCH PiMF-1P1.0 | 1×2×1.0 | 0.5 | 1.3 | 9.7 | 140 |
| MRE-M2XCH PiMF-2P1.0 | 2×2×1.0 | 0.5 | 1.8 | 15.0 | 300 |
| MRE-M2XCH PiMF-3P1.0 | 3×2×1.0 | 0.5 | 1.8 | 15.9 | 360 |
| MRE-M2XCH PiMF-4P1.0 | 4×2×1.0 | 0.5 | 1.9 | 17.0 | 430 |
| MRE-M2XCH PiMF-5P1.0 | 5×2×1.0 | 0.5 | 1.9 | 18.8 | 510 |
| MRE-M2XCH PiMF-6P1.0 | 6×2×1.0 | 0.5 | 2.0 | 20.3 | 600 |
| MRE-M2XCH PiMF-7P1.0 | 7×2×1.0 | 0.5 | 2.0 | 20.3 | 630 |
| MRE-M2XCH PiMF-8P1.0 | 8×2×1.0 | 0.5 | 2.0 | 21.5 | 700 |
| MRE-M2XCH PiMF-10P1.0 | 10×2×1.0 | 0.5 | 2.1 | 24.1 | 860 |
| MRE-M2XCH PiMF-12P1.0 | 12×2×1.0 | 0.5 | 2.2 | 25.2 | 980 |
| MRE-M2XCH PiMF-14P1.0 | 14×2×1.0 | 0.5 | 2.2 | 26.1 | 1080 |
| MRE-M2XCH PiMF-16P1.0 | 16×2×1.0 | 0.5 | 2.3 | 28.2 | 1230 |
| MRE-M2XCH PiMF-19P1.0 | 19×2×1.0 | 0.5 | 2.3 | 30.0 | 1410 |

| Part No. | Construction No. of elements×No. of cores in element×Cross section(mm²) | Nominal Insulation Thickness mm | Nominal Sheath Thickness mm | Nominal Overall Diameter mm | Nominal Weight kg/km |
| --- | --- | --- | --- | --- | --- |
| MRE-M2XCH PiMF-20P1.0 | 20×2×1.0 | 0.5 | 2.3 | 30.0 | 1450 |
| MRE-M2XCH PiMF-24P1.0 | 24×2×1.0 | 0.5 | 2.5 | 34.4 | 1780 |
| MRE-M2XCH PiMF-30P1.0 | 30×2×1.0 | 0.5 | 2.8 | 37.8 | 2220 |
| MRE-M2XCH PiMF-37P1.0 | 37×2×1.0 | 0.5 | 2.9 | 40.5 | 2620 |
| MRE-M2XCH PiMF-1P1.5 | 1×2×1.5 | 0.6 | 1.3 | 10.7 | 170 |
| MRE-M2XCH PiMF-2P1.5 | 2×2×1.5 | 0.6 | 1.9 | 16.9 | 380 |
| MRE-M2XCH PiMF-3P1.5 | 3×2×1.5 | 0.6 | 1.9 | 17.9 | 450 |
| MRE-M2XCH PiMF-4P1.5 | 4×2×1.5 | 0.6 | 1.9 | 19.0 | 530 |
| MRE-M2XCH PiMF-5P1.5 | 5×2×1.5 | 0.6 | 2.0 | 21.3 | 650 |
| MRE-M2XCH PiMF-6P1.5 | 6×2×1.5 | 0.6 | 2.1 | 23.0 | 750 |
| MRE-M2XCH PiMF-7P1.5 | 7×2×1.5 | 0.6 | 2.1 | 23.0 | 800 |
| MRE-M2XCH PiMF-8P1.5 | 8×2×1.5 | 0.6 | 2.1 | 24.4 | 890 |
| MRE-M2XCH PiMF-10P1.5 | 10×2×1.5 | 0.6 | 2.3 | 27.6 | 1110 |
| MRE-M2XCH PiMF-12P1.5 | 12×2×1.5 | 0.6 | 2.3 | 28.7 | 1250 |
| MRE-M2XCH PiMF-14P1.5 | 14×2×1.5 | 0.6 | 2.3 | 29.7 | 1380 |
| MRE-M2XCH PiMF-16P1.5 | 16×2×1.5 | 0.6 | 2.4 | 32.1 | 1580 |
| MRE-M2XCH PiMF-19P1.5 | 19×2×1.5 | 0.6 | 2.5 | 34.4 | 1820 |
| MRE-M2XCH PiMF-20P1.5 | 20×2×1.5 | 0.6 | 2.5 | 34.4 | 1870 |
| MRE-M2XCH PiMF-24P1.5 | 24×2×1.5 | 0.6 | 2.9 | 39.9 | 2400 |
| MRE-M2XCH PiMF-30P1.5 | 30×2×1.5 | 0.6 | 3.0 | 43.3 | 2870 |
| MRE-M2XCH PiMF-37P1.5 | 37×2×1.5 | 0.6 | 3.2 | 46.6 | 3410 |
| MRE-M2XCH TiMF-1T0.75 | 1×3×0.75 | 0.5 | 1.3 | 9.7 | 140 |
| MRE-M2XCH TiMF-2T0.75 | 2×3×0.75 | 0.5 | 1.8 | 15.4 | 320 |
| MRE-M2XCH TiMF-3T0.75 | 3×3×0.75 | 0.5 | 1.8 | 16.2 | 380 |
| MRE-M2XCH TiMF-4T0.75 | 4×3×0.75 | 0.5 | 1.9 | 17.9 | 460 |
| MRE-M2XCH TiMF-5T0.75 | 5×3×0.75 | 0.5 | 2.0 | 19.8 | 560 |
| MRE-M2XCH TiMF-6T0.75 | 6×3×0.75 | 0.5 | 2.1 | 22.2 | 670 |
| MRE-M2XCH TiMF-7T0.75 | 7×3×0.75 | 0.5 | 2.1 | 22.2 | 710 |
| MRE-M2XCH TiMF-8T0.75 | 8×3×0.75 | 0.5 | 2.1 | 23.7 | 800 |
| MRE-M2XCH TiMF-10T0.75 | 10×3×0.75 | 0.5 | 2.2 | 26.7 | 980 |
| MRE-M2XCH TiMF-12T0.75 | 12×3×0.75 | 0.5 | 2.3 | 28.3 | 1120 |
| MRE-M2XCH TiMF-14T0.75 | 14×3×0.75 | 0.5 | 2.3 | 29.4 | 1240 |
| MRE-M2XCH TiMF-16T0.75 | 16×3×0.75 | 0.5 | 2.4 | 31.4 | 1400 |
| MRE-M2XCH TiMF-19T0.75 | 19×3×0.75 | 0.5 | 2.5 | 33.9 | 1620 |
| MRE-M2XCH TiMF-20T0.75 | 20×3×0.75 | 0.5 | 2.5 | 34.4 | 1680 |
| MRE-M2XCH TiMF-24T0.75 | 24×3×0.75 | 0.5 | 2.8 | 37.9 | 2070 |
| MRE-M2XCH TiMF-30T0.75 | 30×3×0.75 | 0.5 | 3.0 | 42.0 | 2520 |
| MRE-M2XCH TiMF-32T0.75 | 32×3×0.75 | 0.5 | 3.0 | 43.4 | 2670 |
| MRE-M2XCH TiMF-1T1.0 | 1×3×1.0 | 0.5 | 1.3 | 10.1 | 160 |

| Part No. | Construction No. of elements×No. of cores in element×Cross section(mm²) | Nominal Insulation Thickness mm | Nominal Sheath Thickness mm | Nominal Overall Diameter mm | Nominal Weight kg/km |
| --- | --- | --- | --- | --- | --- |
| MRE-M2XCH TiMF-2T1.0 | 2×3×1.0 | 0.5 | 1.8 | 16.1 | 360 |
| MRE-M2XCH TiMF-3T1.0 | 3×3×1.0 | 0.5 | 1.9 | 17.3 | 440 |
| MRE-M2XCH TiMF-4T1.0 | 4×3×1.0 | 0.5 | 1.9 | 18.8 | 530 |
| MRE-M2XCH TiMF-5T1.0 | 5×3×1.0 | 0.5 | 2.0 | 20.8 | 640 |
| MRE-M2XCH TiMF-6T1.0 | 6×3×1.0 | 0.5 | 2.1 | 23.4 | 770 |
| MRE-M2XCH TiMF-7T1.0 | 7×3×1.0 | 0.5 | 2.1 | 23.4 | 820 |
| MRE-M2XCH TiMF-8T1.0 | 8×3×1.0 | 0.5 | 2.2 | 25.2 | 930 |
| MRE-M2XCH TiMF-10T1.0 | 10×3×1.0 | 0.5 | 2.3 | 28.4 | 1150 |
| MRE-M2XCH TiMF-12T1.0 | 12×3×1.0 | 0.5 | 2.3 | 29.9 | 1310 |
| MRE-M2XCH TiMF-14T1.0 | 14×3×1.0 | 0.5 | 2.4 | 31.3 | 1470 |
| MRE-M2XCH TiMF-16T1.0 | 16×3×1.0 | 0.5 | 2.5 | 33.4 | 1660 |
| MRE-M2XCH TiMF-19T1.0 | 19×3×1.0 | 0.5 | 2.8 | 36.5 | 2010 |
| MRE-M2XCH TiMF-20T1.0 | 20×3×1.0 | 0.5 | 2.8 | 37.1 | 2090 |
| MRE-M2XCH TiMF-24T1.0 | 24×3×1.0 | 0.5 | 2.9 | 40.3 | 2450 |
| MRE-M2XCH TiMF-30T1.0 | 30×3×1.0 | 0.5 | 3.1 | 44.7 | 3000 |
| MRE-M2XCH TiMF-32T1.0 | 32×3×1.0 | 0.5 | 3.1 | 46.2 | 3180 |
| MRE-M2XCH TiMF-1T1.5 | 1×3×1.5 | 0.6 | 1.4 | 11.4 | 200 |
| MRE-M2XCH TiMF-2T1.5 | 2×3×1.5 | 0.6 | 1.9 | 18.2 | 450 |
| MRE-M2XCH TiMF-3T1.5 | 3×3×1.5 | 0.6 | 2.0 | 19.5 | 560 |
| MRE-M2XCH TiMF-4T1.5 | 4×3×1.5 | 0.6 | 2.0 | 21.3 | 670 |
| MRE-M2XCH TiMF-5T1.5 | 5×3×1.5 | 0.6 | 2.1 | 23.6 | 820 |
| MRE-M2XCH TiMF-6T1.5 | 6×3×1.5 | 0.6 | 2.2 | 26.6 | 990 |
| MRE-M2XCH TiMF-7T1.5 | 7×3×1.5 | 0.6 | 2.2 | 26.6 | 1060 |
| MRE-M2XCH TiMF-8T1.5 | 8×3×1.5 | 0.6 | 2.3 | 28.6 | 1200 |
| MRE-M2XCH TiMF-10T1.5 | 10×3×1.5 | 0.6 | 2.4 | 32.3 | 1480 |
| MRE-M2XCH TiMF-12T1.5 | 12×3×1.5 | 0.6 | 2.5 | 34.3 | 1700 |
| MRE-M2XCH TiMF-14T1.5 | 14×3×1.5 | 0.6 | 2.6 | 35.9 | 1920 |
| MRE-M2XCH TiMF-16T1.5 | 16×3×1.5 | 0.6 | 2.9 | 38.8 | 2260 |
| MRE-M2XCH TiMF-19T1.5 | 19×3×1.5 | 0.6 | 3.0 | 41.8 | 2620 |
| MRE-M2XCH TiMF-20T1.5 | 20×3×1.5 | 0.6 | 3.0 | 42.5 | 2720 |
| MRE-M2XCH TiMF-24T1.5 | 24×3×1.5 | 0.6 | 3.1 | 46.2 | 3200 |
| MRE-M2XCH TiMF-30T1.5 | 30×3×1.5 | 0.6 | 3.3 | 51.2 | 3910 |
| MRE-M2XCH TiMF-32T1.5 | 32×3×1.5 | 0.6 | 3.4 | 53.2 | 4180 |