Date: 19 Sep 2010

Part No.: PS80457

## 1.5MM2 MICA & LSZH INSULATION

## SHIELDED SWA VERMIN-PROOF FRPVC

## FIRE RESISTANT SPEAKER CABLE

Application:

This cable is used for power limited circuit, remote control, signaling, security systems, communications, intercom/P.A. systems, sound/audio, and nurse call.

Construction:

Armoured Type：

Conductor:            1.5mm2 Annealed copper wire conductors class 2.

Insulation:             Mica tape and LSZH insulation

Collective screen:   Aluminium/p.e.t.p. laminated tape applied with the metallic side down in electrical contact with a 0.5mm 2 tinned copper drain wire over the p.e.t.p. binder tape.

Bedding:          FRPVC compound.

Armour:          Steel wire armour

Sheathing:             Vermin-proof FRPVC

Packing:                The cables are delivered on non-returnable wood drums.

Standard:         IEC60332 & IEC60331

Marking:  The cable is marked with white or other colors as follows:

- Caledonian Cables Co., Ltd

- Type and size of cable

- Manufacturing date

- Meter Marking

The meter mark is printed every meter with an accuracy of 0.5%.

Other printing are available according to customer request.

Table 1. Technical Data

| Number of Pairs | Conductor Size (mm2) | Dia.of steel wire armour (mm) | Nominal OD (mm) |
| --- | --- | --- | --- |
| 3 | 1.5 | 1.25 | 25 |

The data included in the present catalogue are merely indicative; Caledonian Cables Limited reserves to itself the right to change them as its own discretion in any time.